Product code: L1001		Available containers: P1001

Description:		1 Litre rectangular lid

Lid type:		Standard compatible with tamper evident container

Lid Dimensions:	Length	184mm +/- 1mm

Width	119.9mm +/- 1mm

Height	6mm+/- 1mm

Weight	15g +/- 5%

Material:		Polypropylene, widely recyclable & suitable for direct food contact.

Suitable for domestic freezing, however care must be taken when handling.

Recommended filling at ambient or chilled temperatures.

Intended use for dairy products.

Colours:		Natural, white, black & purple

Other colours are possible upon request.

Decoration:		In mould and post mould labels are available upon request.

Packing:		630 units per box

4 boxes per layer

5 layers per pallet

Box dimensions 	552 x 400 x 324mm

Pallet Dimensions:	Length 1200mm

Width 800mm

Height 144mm

Weight 12-21kgs

Stacking:	Maximum 1 pallet high. Customers should make internal trials to determine the viability of stacking and transportation once applied to filled containers.

Quality:	Deviations in the product specifications of 0.025% (i.e. 25 items per 100,000) are considered acceptable in the industry and therefore cannot be construed as defect.

Works orders including full traceability information will be held for a minimum period of twelve months from the date of manufacture.

The material complies with the material and articles in contact regulation (EC)1935/2004, food contact plastics regulation (EU) 10/2011 and regulation (EC) 2023/2006 good manufacturing practices.

The user of the product is responsible for ensuring that the finished food packaging complies with the applicable migration limits in the food itself under actual conditions of use.

Issue 7       22/07/2019

Technical Data Sheet

1 Litre rectangular lid